AN ALTERNATIVE GRIND FOR THE SKEW CHISEL

Alan Lacer

Geometry:  70 degrees from point to point; ground bevel angle (equivalent to bevel length) is expressed as 1.5 x steel thickness; behind the short point’s bevel a full round to the end of the blade; behind the long point’s bevel a flattened surface but with the corners removed.

ADVATAGES:

- “Tells” you where to cut for planing and rolling cuts (cut where curved)

- For planning and rolling cuts gives increased clearance angle for the trailing point and trailing edge

- Gives between 15% and 20% more cutting edge

- For peeling cuts is self-limiting

- For peeling cuts puts the handle at a right angle to the axis of lathe

- Straight section used for scraping actions

- Straight section used for finishing cuts on rounded pommels

- Curved edge works better on “chippy” or figured woods

9.  Curved edge has lower impact when planning or rolling, has a progressive     entry into the wood, better at shearing the fibers

10.  Curved edge works better for concave cuts (a curve fits a curve better)

DISADVANTAGE:   Harder to sharpen than a traditional ground skew.

For sharpening two sizes of skews with this grind see Alan on YouTube:

https://www.youtube.com/watch?v=dmCxDToHm6Y&t=19s